SDK Troubleshooting Guide				      2/27/2024

Prerequisites:

- Set up your authentication server as described in the “AliveCor SDK overview” using the credentials provided to you by AliveCor.  The Section “SDK Authentication using the Kardia SDK Auth Server:” is the section that contains the details on setting up your authentication server.

- It is always best to make sure authentication works by using the AliveCor provided sample app as a starting point.  You will need to modify the sample app by changing the bundleid and the auth server IP address, as well as the hard coded values for partnerid, and teamid.  The AliveCor provided sample app has pre-populated values for these in the token request code that you should not use.  Once you replace the bundleid, auth server URL, and partner credentials with your values, you can verify authentication using the sample app.

- If you are having issues, install Charles web proxy (a free tool) or other logging tool on your PC and configure Charles per these instructions.  https://www.charlesproxy.com/

  - The target device will need to have its IP settings modified as well, per these instructions

  - https://docs.tealium.com/platforms/ios-swift/charles-proxy-iosdevice/

  - https://docs.tealium.com/platforms/android-kotlin/charles-proxy-android/

- Logs from Charles (or a similar tool) will be needed if you need to enter a support ticket for authentication issues

- Make sure you have an account on the AliveCor developers portal.  Contact your AliveCor representative if this has not been set up.  The URL for opening support tickets is https://alivecor.atlassian.net/servicedesk/customer/portal/4

Step 1: Getting Authentication to work:

If you have followed the steps above, the normal sequence of events is as follows:

- Your app requests a token from http://x.x.x.x:8080/token (x.x.x.x is the IP address of the auth server you set up in prerequisite #1.  The post looks like this:

POST body will contain 4 values. “patientMrn” can be any alphanumeric character up to 32 characters in length.  The other 3 are provided to you as part of your credentials.

{

"bundleId": "bid1",

"partnerId": "examplePartnerId",

"patientMrn": "ExamplePatient MRN",

"teamId": "exampleTeamId"

}

The JWT token is the normal response.  If you are not getting a token in response, open a support ticket and include the Charles logs from the /token post and response.

- You initialize your SDK using the token you received after the above post, using the commands listed in the iOS or Android reference documents.

- During initialization, the SDK automatically posts to the AliveCor cloud at https://us-kardia-staging.alivecor.com/auth/sdkPatient (or for production, it is https://us-kardia.alivecor.com/auth/sdkPatient)

- An example of a successful post/response is shown below.  The response contains information about what devices your SDK is allowed to use, the expiration of the authentication, and the phone home frequency, which is how long before you need to re-authenticate the SDK.

POST:

{

"sdkVersion": "1.2.0",

"deviceId": "A97560D7-9BB6-4921-8295-555A22243191"

}

RESPONSE:

{

"features": [{

"publicId": "fid2",

"featureName": "device-kardia-triangle",

"createdAt": "2020-12-11T04:37:05.052914Z",

"updatedAt": "2020-12-11T04:37:05.052914Z"

}, {

"publicId": "fid3",

"featureName": "device-kardia-mobile",

"createdAt": "2020-10-15T21:58:48.684409Z",

"updatedAt": "2020-10-15T21:58:48.684409Z"

}],

"expiration": "2022-06-22T17:46:55.125474425Z",

"partnerPhoneHomeFrequency": 604800

}

- If you are unable to get this far, please open a support ticket and send AliveCor the Charles logs from the /token post and response, as well as the SDKs automatic post to https://us-kardia-staging.alivecor.com/auth/sdkPatient  so we can troubleshoot.  Charles can be configured to only log transactions to certain IP addresses, so you can filter out all the IP traffic to/from the target device that does not apply, to ensure you only see the two transactions described above.

Step 2:  Initialization failures with your app

Once the sample app is authenticating and taking recordings, you can start to build your app, using as many of the sample app components as you like.  If you are able to initialize the sample app but your project will not initialize, here are some troubleshooting steps to take:

- Look at the Charles logs to confirm the posts and responses to x.x.x.x/token and https://us-kardia-staging.alivecor.com/auth/sdkPatient  are working the same in your app as they did for the sample app.

- If the authentication looks correct but you still cannot initialize your app, you can open a support ticket with AliveCor and include the following information:

  - Details about the target device, including SDK version, phone (or tablet) make and model number, OS version

  - A code snippet or your entire source code that can reproduce the initialization failure

  - The run time logs that show all the errors during initialization

Step 3:  Other failures or issues with your app

- You can open a support ticket with AliveCor and include the following information:

  - Details about the target device, including phone (or tablet) make and model number, OS version

  - The AliveCor SDK version you are using.

  - A code snippet or your entire source code that can reproduce the problem.  If is also occurring in the sample app let us know that as well.

  - The run time logs that show all the errors during failure (if any)

  - A full description of the failure.  Screen shots or video capture are very helpful

Other things to be aware of:

Make sure to set the AC mains frequency of the recording flow to 50 or 60Hz, depending on the region.  The sample app is set for 60Hz.  In Europe the correct value is 50Hz, in the US it is 60Hz.  Other countries will use one or the other.  If the mains filter is set to the wrong value, you will get interference in the recordings that looks like a clean 50Hz or 60Hz sine wave superimposed on the recording.